VXLAN Complete Study Guide

10-20 Minute Comprehensive Review

# 1. WHAT IS VXLAN? (The 5-Minute Explanation)

VXLAN = Virtual eXtensible LAN. It wraps (encapsulates) a full Ethernet frame inside a UDP packet so it can travel across a regular IP network. Think of it as putting a letter (your VM traffic) inside a bigger envelope (UDP/IP) to ship it across the internet.

## The Core Analogy

- Underlay = The postal road network (trucks, highways, sorting centers)

- Overlay = The letters being delivered — sender/receiver don't care which truck

- VTEP = The post office that puts letters into envelopes and opens them at delivery

- VNI = The department/mailbox number that separates one tenant's mail from another

## Why Was VXLAN Created?

| VLAN Problem | VXLAN Solution |
| --- | --- |
| Only 4,094 VLANs (12-bit) | 16.7 million VNIs (24-bit) |
| STP blocks redundant links | Runs over L3 — ECMP uses all links |
| Cannot cross Layer 3 routers | Tunnels L2 over L3 natively |
| VM migration breaks IP | VMs keep IP/MAC through tunnel |
| No overlapping IP support | Each VNI is isolated namespace |

# 2. PACKET FORMAT — The 50-Byte Overhead

Every VXLAN packet adds exactly 50 bytes of headers around your original frame:

| Header | Size | Key Field / Purpose |
| --- | --- | --- |
| Outer Ethernet | 14B | Next-hop MAC (ToR switch). Rewritten at every hop. |
| Outer IP | 20B | Src = VTEP-A IP, Dst = VTEP-B IP. DF=1. DSCP copied from inner. |
| Outer UDP | 8B | Dst port 4789 (IANA). Src port = hash(inner 5-tuple) for ECMP. |
| VXLAN Header | 8B | I flag=1 (valid VNI). VNI = 24-bit tenant segment ID. |
| Inner Ethernet Frame | var | Original VM frame — preserved byte-for-byte. Inner TTL unchanged. |

MTU Rule: Switch MTU must be ≥ 1600 bytes (standard) or 9100 bytes (jumbo). Wrong MTU = silent black holes.

# 3. KEY COMPONENTS

## VTEP — VXLAN Tunnel Endpoint

The VTEP is the device (or software) that encapsulates and decapsulates VXLAN traffic. It's the post office in our analogy.

- Identified by its loopback IP address (stable, survives link failures)

- Software VTEP: runs in hypervisor (VMware NSX, Linux kernel, OVS, Flannel)

- Hardware VTEP: runs in switch ASIC at line rate (Cisco Nexus, Arista)

- Per-VNI MAC table: each VNI has its own separate forwarding table

## VNI — VXLAN Network Identifier

| Range | 1 to 16,777,215 (24-bit = 2^24) |
| --- | --- |
| Purpose | Identifies which logical network segment a packet belongs to |
| Isolation | VTEP has separate MAC table per VNI — traffic never crosses VNI boundary |
| vs VLAN | 4,094 VLANs vs 16.7 million VNIs — 4,096x more capacity |
| L2VNI | Bridges a Layer 2 subnet (one per subnet) |
| L3VNI | Transit VNI for routing between VNIs — one per tenant VRF |

## Underlay vs Overlay

| Underlay | Physical IP network. Knows nothing about VNIs or VMs. Runs OSPF/IS-IS/eBGP + ECMP. |
| --- | --- |
| Overlay | Virtual L2 networks riding on top. VMs see a flat Ethernet. No physical topology awareness. |
| VTEP | The boundary between them. Encapsulates (overlay→underlay) and decapsulates (underlay→overlay). |
| Decoupling benefit | Physical network can change without reconfiguring VMs. New VNIs need no physical changes. |

# 4. HOW TRAFFIC FLOWS (2-Minute Overview)

## Same VNI — Different Hosts

- VM-A sends frame to VM-B (MAC known from BGP EVPN)

- VTEP-A wraps frame: adds VXLAN(VNI=5001) + UDP(dst=4789) + IP(src=VTEP-A, dst=VTEP-B) + Ethernet

- Spine switches forward outer IP packet via ECMP — they never look inside

- VTEP-B strips all outer headers, reads VNI=5001, delivers inner frame to VM-B

- VM-B receives identical frame — unaware of VXLAN tunnel

## Different VNI (Inter-VNI Routing — Symmetric IRB)

- VM-A (VNI 5001) sends to VM-C (VNI 5002) — different subnets

- Frame goes to anycast GW MAC (same IP/MAC on every VTEP)

- VTEP-A routes in tenant VRF, stamps L3VNI (e.g., 9001) as transit VNI

- Inner Ethernet uses VTEP Router MACs (RMACs) — not VM MACs

- VTEP-C receives L3VNI 9001 → looks up VRF → routes to VNI 5002 → delivers to VM-C

- TTL decremented TWICE (once per routing VTEP) — VM-C sees TTL-2

## Same VNI — Unknown Destination (BUM Traffic)

- BUM = Broadcast + Unknown Unicast + Multicast

- With flood-and-learn: flood to all VTEPs in VNI via multicast or ingress replication

- With BGP EVPN: ARP suppressed at local VTEP — zero flooding needed for known MACs

# 5. CONTROL PLANE: FLOOD-AND-LEARN vs BGP EVPN

## Flood-and-Learn (Legacy)

- Data plane only — no separate control protocol

- Unknown MAC? Flood to all VTEPs in VNI

- Learn from responses: inner src MAC + outer src IP = MAC→VTEP mapping

- ARP = broadcast → floods entire fabric constantly

- VM migration: GARP (Gratuitous ARP) may be missed → stale entries up to 300 seconds

- Requires multicast (PIM/RP) in underlay for efficient BUM delivery

Problems: BUM floods scale with N², poor mobility, no topology awareness, needs multicast

## BGP EVPN (Modern Standard)

- BGP with L2VPN EVPN address family (AFI 25, SAFI 70) as control plane

- VTEPs peer with Route Reflectors (spines) — not full-mesh (avoids O(N²) sessions)

- MAC/IP learned proactively via RT-2 routes — before any traffic flows

- ARP suppressed: VTEP answers ARP from local cache (RT-2 has IP+MAC)

- VM migration: BGP withdraw old RT-2, advertise new RT-2 with seq+1 → converges in <10ms

- No multicast needed: ingress replication list built from RT-3 routes automatically

## EVPN Route Types — Quick Reference

| Type | Name | Purpose / What It Does |
| --- | --- | --- |
| RT-1 | Ethernet Auto-Discovery | Multi-homing: mass withdrawal on failure, aliasing for load balancing |
| RT-2 | MAC/IP Advertisement | THE MOST IMPORTANT. Distributes MAC + IP + L2VNI + L3VNI. Enables MAC learning + ARP suppression + VM mobility + host routes in VRF |
| RT-3 | Inclusive Multicast ET | VTEP announces 'I participate in VNI X'. Builds ingress replication lists. Replaces multicast. |
| RT-4 | Ethernet Segment | Multi-homing: Designated Forwarder election. Prevents duplicate BUM delivery to dual-homed server |
| RT-5 | IP Prefix | Subnet routes for inter-VNI routing and external prefix import. One route per subnet (vs RT-2 per host) |

# 6. ARP SUPPRESSION

One of BGP EVPN's most impactful features. Instead of flooding ARP across the fabric, each VTEP answers ARP locally.

## How It Works

- RT-2 routes carry BOTH MAC and IP — VTEP builds ARP cache from these

- VM ARPs for 10.1.1.1 → Local VTEP checks cache → already knows it → answers directly

- Zero ARP packets cross the fabric — VM-A (the target) never even wakes up

## Scale Impact

| Without suppression | 10,000 VMs × 1 ARP/60s × 999 remote VTEPs = 166,833 packets/sec of pure ARP |
| --- | --- |
| With suppression | ~0 ARP packets cross the fabric — all answered locally |
| Reduction | 95%+ of BUM traffic eliminated instantly |

# 7. VM LIVE MIGRATION IN VXLAN

## With Flood-and-Learn (Bad)

- VM-A moves from VTEP-A to VTEP-G

- VTEP-G sends Gratuitous ARP (GARP) — may be missed by some VTEPs

- VTEPs that missed GARP: still send traffic to VTEP-A → traffic blackholed

- Recovery: stale entries age out after 300 seconds (5 minutes!)

- Max blackhole window: 300 seconds

## With BGP EVPN (Good)

- VM-A activates on VTEP-G

- VTEP-G advertises RT-2: MAC-A, IP-A, via VTEP-G, seq=1 (seq+1 proves it's newer)

- Route Reflector distributes to ALL VTEPs within ~10ms

- All VTEPs update: MAC-A → VTEP-G (MAC Mobility seq prevents conflicts)

- VTEP-A withdraws its RT-2 for MAC-A

- Total blackhole window: ~10 milliseconds

## The MAC Mobility Sequence Number

- Every RT-2 carries a sequence number (starts at 0)

- Migration increments it: 0 → 1 → 2...

- Higher sequence number always wins — no confusion about which VTEP is current

- Sticky flag (bit=1): marks physical servers that should never move — alerts if MAC moves

# 8. HOW SAME IPs DON'T CONFLICT

This is pure namespace isolation — like how two apartments in different buildings can both be 'unit 101'.

## Layer 2 Isolation (VNI)

- Tenant A uses 10.0.0.0/24 in VNI 5001

- Tenant B uses 10.0.0.0/24 in VNI 6001

- Each VNI has its own separate MAC table — lookup in VNI 5001 never touches VNI 6001 table

- ARP in VNI 5001 never reaches VMs in VNI 6001

## Layer 3 Isolation (VRF)

- Each tenant gets its own VRF (Virtual Routing and Forwarding instance)

- VRF-TenantA and VRF-TenantB have completely separate routing tables

- Both have 10.0.0.0/24 in their routing table — no conflict because different tables

- BGP Route Targets control which VTEPs import which routes — Tenant B routes never enter Tenant A VRF

## Summary of Isolation Stack

| L2: VNI | Separate broadcast domain per VNI — MAC tables isolated |
| --- | --- |
| L3: VRF | Separate routing table per tenant — overlapping IPs coexist |
| Control: Route Targets | BGP EVPN RT policy — Tenant B routes never imported into Tenant A VRF |
| Transit: L3VNI | Per-tenant transit VNI for routing — traffic of different tenants never mixes |

# 9. SYMMETRIC vs ASYMMETRIC IRB

IRB = Integrated Routing and Bridging. How VTEPs route between VNIs (different subnets).

| Asymmetric IRB | Ingress VTEP does ALL routing. Stamps destination VNI in VXLAN header. Egress VTEP only bridges. Problem: every VTEP must have ALL VNIs configured — doesn't scale. |
| --- | --- |
| Symmetric IRB | BOTH ingress AND egress VTEP route. Uses L3VNI as transit. Each VTEP only needs its own VNIs + one L3VNI per VRF. Scales to thousands of VNIs. Industry standard. |
| L3VNI | Transit VNI carrying routed traffic between VTEPs. One per tenant VRF. Inner Ethernet uses VTEP Router MACs (RMACs) not VM MACs. |
| Anycast Gateway | Same gateway IP + MAC configured on EVERY VTEP. VM's default gateway is always local VTEP — zero reconfiguration on migration. No VRRP/HSRP needed. |

# 10. MULTICAST vs HEAD-END REPLICATION

Two ways to deliver BUM traffic to all VTEPs in a VNI when you don't have BGP EVPN (or for residual BUM with EVPN):

| Multicast | Source sends 1 copy to multicast group (e.g., 239.1.1.1 for VNI 5001). PIM infrastructure replicates to all subscribers. Efficient but requires PIM + RP in underlay. RP = single point of failure. |
| --- | --- |
| Head-End Replication (Ingress Replication) | Source VTEP sends N separate unicast copies (one per remote VTEP). No multicast needed — pure IP. List of VTEPs built automatically from RT-3 routes with BGP EVPN. Source CPU scales with N VTEPs. |
| Modern best practice | BGP EVPN + Head-End Replication. EVPN suppresses 95%+ of BUM → residual BUM handled cheaply by ingress replication. No multicast needed at all. |

# 11. VXLAN DESIGN: SPINE-LEAF + ECMP

## Spine-Leaf Topology

- Every Leaf connects to every Spine — no Leaf-to-Leaf or Spine-to-Spine links

- Any server to any server = exactly 2 hops (always predictable latency)

- No Spanning Tree Protocol — pure Layer 3 between switches

- Add spines → more bandwidth. Add leaves → more ports. Linear scaling, zero disruption.

- Spines: pure IP forwarding, often act as BGP EVPN Route Reflectors

- Leaves: are the VTEPs. Connect to servers and perform VXLAN encap/decap.

## Why ECMP Is Critical

- Without ECMP: all VXLAN traffic between VTEP pair uses ONE spine (others idle)

- With ECMP: flows distributed across all spines simultaneously — full bandwidth

- ECMP hashes on outer 5-tuple. Key: outer UDP source port varies per inner flow

- UDP src port = hash(inner src IP + dst IP + src port + dst port + protocol)

- Same inner flow → same outer src port → same spine (no TCP reordering)

- Different inner flows → different outer src ports → different spines (load balanced)

# 12. SECURITY

## What VXLAN Provides

- Tenant isolation via VNI (L2 boundary)

- Routing isolation via VRF (L3 boundary)

- With BGP EVPN: ARP binding validation, MAC mobility detection

## What VXLAN Does NOT Provide

- Encryption: all traffic is plain text — anyone on underlay can read it

- VTEP authentication: no certificate exchange — any device can send VXLAN on UDP/4789

- Integrity protection: no tamper detection on packets

## How to Secure VXLAN

| Underlay ACLs | Whitelist: only known VTEP IPs can send VXLAN (UDP 4789) |
| --- | --- |
| MACsec | Encrypts underlay links hop-by-hop at line rate. Hardware ASIC — zero performance penalty. |
| IPsec (VTEP-to-VTEP) | End-to-end encryption. IKEv2 + certificates authenticate each VTEP. Spines see only encrypted ESP payload. |
| BGP Auth | MD5 + GTSM on all BGP EVPN sessions. Route filtering + prefix limits prevent route injection. |
| Distributed Firewall | VMware NSX, Cilium, Calico: policy at VM vNIC level. East-west security without hairpinning. |
| Application TLS | Zero-trust: encrypt at app layer regardless of network. Works even if VXLAN compromised. |

# 13. VXLAN IN KUBERNETES

## Why Kubernetes Uses VXLAN

- Every pod needs unique IP, any pod talks to any pod (flat L3 network)

- Nodes on different subnets/AZs — physical network doesn't have pod routes

- VXLAN overlay: nodes act as VTEPs, pods are VMs, VNI typically = 1 per cluster

## Common CNI Plugins

| Flannel | Simplest. VXLAN backend (VNI=1). Port 8472. No network policy. etcd for control plane. |
| --- | --- |
| Calico | BGP-native routing preferred. VXLAN as fallback. Port 4789. Rich NetworkPolicy. Per-pod /32 routes. |
| Cilium | eBPF-based VXLAN processing in kernel. Fastest. L7 policy. Hubble observability. WireGuard encryption. |
| Weave | VXLAN-based. Simple setup. Slower than Cilium. |

## Key Difference: K8s vs DC VXLAN

- K8s: typically 1 VNI per cluster, control plane = K8s API / etcd

- Data Center: thousands of VNIs, control plane = BGP EVPN

- Same VXLAN encapsulation mechanism — different scale and control plane

# 14. TROUBLESHOOTING QUICK REFERENCE

## Common Issues & Checks

| Symptom | What To Check |
| --- | --- |
| Small pings work, large transfers fail / hang | MTU! Switch MTU must be ≥ 1600 (standard) or 9100 (jumbo). Check for ICMP type 3 code 4 being filtered. Classic black hole pattern. |
| VMs can't reach other VMs same VNI | BGP EVPN session up? RT-2 routes for both VMs present? VTEP IP reachable via underlay? Outer VTEP loopbacks reachable (ping VTEP-to-VTEP)? |
| EVPN routes not being learned | BGP session established? Correct address-family (l2vpn evpn) activated? Route Targets match? Max-prefix limit hit? RR reachable? |
| MAC not learned / traffic flooding | RT-2 being advertised by correct VTEP? ARP suppression enabled? MAC table not full? BGP RT import policy correct? |
| Traffic blackhole after VM migration | BGP EVPN: RT-2 with new VTEP + higher seq# propagated? Flood-and-learn: GARP sent? Stale entries — wait or clear MAC table. |
| Broadcast storm in VXLAN | With EVPN: ARP suppression working? RT-2 covering all VMs? Without EVPN: check for rogue VM sending random MACs (MAC flooding attack). |
| Inter-VNI routing not working | L3VNI configured on ALL VTEPs? Anycast GW same IP+MAC everywhere? VRF routing table has correct RT-5 or RT-2 host routes? RMAC advertised? |

## Key Commands (Generic)

show bgp l2vpn evpn summary          — all BGP EVPN sessions + status

show bgp l2vpn evpn route-type 2     — all MAC/IP routes in fabric

show bgp l2vpn evpn route-type 3     — VTEP membership per VNI

show bgp l2vpn evpn route-type 5     — subnet/prefix routes (inter-VNI)

show vxlan vtep                       — all known remote VTEPs

show vxlan vni                        — VNI table on this VTEP

show vxlan address-table              — MAC table for VXLANshow ip route vrf VRF-TenantA         — tenant routing table

ping vrf VRF-TenantA 10.2.2.1        — test inter-VNI routing

tcpdump -i eth0 udp port 4789 -w cap — capture VXLAN packets

ping -s 1450 -M do <remote-ip>       — test for MTU issues (DF bit set)

# 15. VXLAN vs GENEVE vs NVGRE — Quick Compare

| VXLAN (RFC 7348) | UDP/4789. 8-byte header. 24-bit VNI. Most widely deployed. BGP EVPN control plane. No native extensibility. |
| --- | --- |
| VXLAN-GPE | VXLAN Generic Protocol Extension. Adds next-protocol field. Supports NSH (Network Service Header) for service chaining. Used in SFC/NFV. |
| Geneve (RFC 8926) | UDP/6081. Variable-length header with TLV options. More flexible than VXLAN. Used by OVN (Open Virtual Network), AWS Nitro. |
| NVGRE | Uses GRE (protocol 47 not UDP). Microsoft's protocol. No ECMP without entropy extensions. Largely replaced by VXLAN. |

# 16. MIGRATING FROM VLAN TO VXLAN

## Why Migrate

- Exhausted 4,094 VLAN limit

- Need VM mobility across Layer 3 boundaries

- Need multi-tenant isolation with overlapping IPs

- STP blocking bandwidth on redundant links

## Migration Strategy (Phased)

- Phase 1: Deploy spine-leaf underlay. Configure OSPF/eBGP. Verify VTEP loopback reachability.

- Phase 2: Configure BGP EVPN on all VTEPs. Establish sessions to Route Reflectors.

- Phase 3: Map existing VLANs to VNIs (VLAN 100 → VNI 10100). Configure L2VNIs.

- Phase 4: Enable VXLAN on edge VTEPs. VMs transparently moved to VXLAN.

- Phase 5: Configure L3VNI + Anycast Gateway. Enable inter-VNI routing.

- Phase 6: Decommission old VLAN infrastructure.

Key: VXLAN data plane doesn't change. Only control plane changes when adding BGP EVPN. No VM reconfiguration needed.

# 17. DESIGN CHECKLIST — PRODUCTION VXLAN

## Must-Have Configurations

- MTU: 9100 on ALL switch ports (never forget uplinks). Verify end-to-end with large pings.

- BGP EVPN: Minimum 2 Route Reflectors (both spines). Each VTEP peers with both.

- BFD: 50ms × 3 = 150ms failure detection on all BGP sessions and IGP adjacencies.

- ARP Suppression: Enable on all VTEPs. Verify RT-2 routes have IP field included.

- L3VNI: One per tenant VRF. Configure on ALL VTEPs even those without VMs in that VRF.

- Anycast Gateway: Exact same IP + MAC on every VTEP. Use automation — one mistake = VM migration breaks.

- ECMP: Hash must include UDP src port. Verify distribution across all spines.

- VNI Allocation: Document in IPAM. Use randomized (not sequential) VNIs for security.

## Common Mistakes to Avoid

- MTU only on downlinks — forgot uplinks → cross-rack traffic breaks

- Single Route Reflector → entire fabric loses EVPN on spine failure

- Different anycast MAC on some VTEPs → VM migration breaks ARP cache

- No L3VNI → inter-VNI routing fails silently

- ICMP type 3 filtered → MTU black holes invisible and undiagnosable

- Sequential VNI assignment → easy to scan/spoof (security)

VXLAN = Tunnel (data plane)  |  BGP EVPN = Intelligence (control plane)  |  Spine-Leaf = Foundation (physical)

Neither works optimally without the other. Together they are the universal standard for modern data center networking.